___________________________________________________________________________

Faculty of Information Technology and Engineering

Electrical Circuit Lab

| Students ID: 202110795, 202112300, 202110451 | Students Name: Sama Haitham Sammar, Arein Zaid Alkilani, Yara Jehad Rabaya |
| --- | --- |
| Experiment #: 7 | Experiment Name: Steady State Response and Phase Angle of 1St Order Systems |
| Section: 3 | Supervisor Name: Dr Amjad Abu Jazar |
| Date: 30 May 2024 | Day: Thursday |

Objective:

- To study the voltage-current and impedance relationships of a first order system.

- To study phase angle measurement techniques and its relationship with the frequency.

- To study the relationship between time-shift and phase shift.

Apparatus Required:

- Digital Multimeter.

- Signal Generator.

- Oscilloscope.

- Components (Resistors, Capacitors, Inductors, Cables and Breadboard).

Theory and Background:

The voltage and current of a first order system excited by a sinusoidal signal can be represented by two phasors that are not in phase. In this experiment we will study the steady-state response and the phase angle between voltage and current of a first order system.

To study the voltage current relationships of a first order system let’s try to find VR in figure 1.

Figure 1

Apply voltage divider to find the voltage across R:

VR=RR+1jWCVs∠0o

VR=jWRC1+jWRCVs∠0o

.

VR=WRC1+(WRC)2Vs

∠VR=90-tan-1(WRC)

VR=VR∠VR=WRC1+(WRC)2Vs∠(90-tan-1(WRC))

The phase deferent between voltage and current for this arrangement is

θ=90-tan-1(WRC)

- Phasor

The complex voltage or current at a given frequency is characterized by only two parameters, amplitude and phase angle. For example, the complex voltage                                       can be exactly defined if Vm and θ are known. Therefore, for any linear circuit operating in the sinusoidal steady state at a single frequency Ѡ, every current or voltage may be characterized completely by its amplitude and phase angle; and their complex representation will contain the same factor ejwt.

Thus, it is possible to simplify the complex representation by omitting the factor ejwt. Also, additional time and effort may be saved if these complex quantities are written in polar form.

Let us define impedance Z as the ratio of the phasor voltage to phasor current. Impedance is a complex quantity having dimensions of ohms. Think of an inductor L as being represented in the time-domain by its inductance L and in the frequency domain by its impedance j Ѡ L. The same can be said about C and 1/ j Ѡ L. Also, note that the impedance of an inductor and capacitor are functions of frequency.

Table (1):

| Component | Voltage | Phase | Impedance |
| --- | --- | --- | --- |
| R |  | In phase |  |
| C |  | Lead 90 |  |
| L |  | Lag 90 |  |

- Phase Measurement

Phase comparison of two signals of the same frequency can be made using the dual trace feature of the scopes in the lab. To make the comparison, proceed as follows:

- Use the Oscilloscope prop to find the time difference between the two waves (∆t) i.e.: two successive peak or two successive zeroes.

- Use this equation to convert the time delay to degree

θ=Δt*360o*f=Δt*360oT

Where f is the frequency of the signals.

Figure 2

Experiment Procedure:

  - Connect the circuit shown below.

Figure 3

  - Find the RMS voltage across the source, resistor and capacitor for each of the following frequencies (Hz): 100, 500, 1K, 2K, and 5K, 10K,20K,30k.

  - Measure the phase between the source voltage and the circuit current for each of the frequencies listed in step 2.

  - Tabulate your result in the Table (2):

Table (2):

| Freq. (Hz) | ¦Vin¦ By Multisim | ¦Vin¦ on Lab | ¦VR1¦ By Multisim | ¦VR1¦ on Lab | ¦VC1¦ By Multisim | ¦VC1¦ on Lab | Ө By Multisim | Ө on Lab |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 500 | 3.5 V | 3.77V | 362m | 0.41V | 3.5V | 3.74 V | 87.5 | 81 |
| 1k | 3.5 V | 3.8V | 712m | 0.88V | 3.42V | 3.60 V | 85.4 | 73.4 |
| 2k | 3.5 V | 3.7V | 1.43V | 1.61V | 3.23V | 3.42 V | 72.7 | 65.1 |
| 5k | 3.5 V | 3.59V | 2.52V | 2.66V | 2.43V | 2.35 V | 56.16 | 40.4 |
| 10k | 3.5 V | 2.98V | 3.15V | 2.70V | 1.52V | 1.15 V | 31.46 | 24.7 |
| 20k | 3.5 V | 2.3V | 3.4V | 2.20V | 817 mV | 0.40 V | 12.6 | 12.7 |
| 30k | 3.5 V | 1.93V | 3.46V | 1.90V | 553mV | 0.13 V | 8.1 | 8.321 |

Vout leads Vin in all frequancy

By Multisim

500K:

1K:

2K:

5K:

10K:

20K:

30K:

  - Discuss the result in table 2 and support your answer by drawing VOUT (t).

As we see in the table 2 : as we increase the frequency Vr increase and Vc decrease ,

So, we concluded that frequency is inversely proportional to capacitor voltage according to the relation

Xc=1/2πfc,

And as it shown in the multisim photos, so VR increase and Vc decrease so vin remain the same value

Conclusion and Analysis:

- Make a plot of |VC1/VIN| and |VR1/VIN| versus frequency for the collected data.

| frequancy | ¦VC1/VIN¦ | ¦VR1/VIN¦ |
| --- | --- | --- |
| 500 | 1 | 0.109 |
| 1000 | 0.98 | 0.23 |
| 2000 | 0.92 | 0.41 |
| 5000 | 0.69 | 0.75 |
| 10000 | 0.43 | 0.92 |
| 20000 | 0.23 | 0.99 |
| 30000 | 0.158 | 1 |

- Plot the phase between the source voltage and the circuit current as a function of frequency.

| frequancy | phase angle |
| --- | --- |
| 500 | 87.5 |
| 1000 | 85.4 |
| 2000 | 72.7 |
| 5000 | 56.16 |
| 10000 | 31.46 |
| 20000 | 12.6 |
| 30000 | 8.1 |

- Compare the measured phase difference for the calculated RC phase angles and the actual value.

| Freq. (Hz) | Ө By Multisim | Ө on Lab | ¦Ө By Multisim - Ө on Lab¦ |
| --- | --- | --- | --- |
| 500 | 87.5 | 81 | 6.5 |
| 1k | 85.4 | 73.4 | 12.5 |
| 2k | 72.7 | 65.1 | 7.6 |
| 5k | 56.16 | 40.4 | 15.76 |
| 10k | 31.46 | 24.7 | 6.76 |
| 20k | 12.6 | 12.7 | 0.1 |
| 30k | 8.1 | 8.321 | 0.22 |